A.4.25	SMS/CBS 7-bit Santali (Ol Chiki script) Alphabet (0x25)

A.4.25.1	SMS/CBS 7-bit Santali (Ol Chiki script)Alphabet (0x25) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | − 2212 | SP 0020 | 0 0030 | ᱚ 1C5A | ᱪ 1C6A |  | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | ᱛ 1C5B | ᱫ 1C6B | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 |  | " 0022 | 2 0032 | ᱜ 1C5C | ᱬ 1C6C | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | # 0023 | 3 0033 | ᱝ 1C5D | ᱭ 1C6D | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 |  | € 20AC | 4 0034 | ᱞ 1C5E | ᱮ 1C6E | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 |  |  | % 0025 | 5 0035 | ᱟ 1C5F | ᱯ 1C6F | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 |  |  | & 0026 | 6 0036 | ᱠ 1C60 | ᱰ 1C70 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 |  |  | ' 0027 | 7 0037 | ᱡ 1C61 | ᱱ 1C71 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 |  |  | ( 0028 | 8 0038 | ᱢ 1C62 | ᱲ 1C72 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 |  |  | ) 0029 | 9 0039 | ᱣ 1C63 | ᱳ 1C73 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A |  | * 002A | : 003A | ᱤ 1C64 | ᱴ 1C74 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B |  | SS2 ---- | + 002B | ; 003B | ᱥ 1C65 | ᱵ 1C75 | k 006B | ᱸ 1C78 |
| 1 | 1 | 0 | 0 | C |  |  | , 002C | < 003C | ᱦ 1C66 | ᱶ 1C76 | l 006C | ᱹ 1C79 |
| 1 | 1 | 0 | 1 | D | CR 000D |  | - 002D | = 003D | ᱧ 1C67 | ᱷ 1C77 | m 006D | ᱺ 1C7A |
| 1 | 1 | 1 | 0 | E |  |  | . 002E | > 003E | ᱨ 1C68 |  | n 006E | ᱻ 1C7B |
| 1 | 1 | 1 | 1 | F |  |  | / 002F | ? 003F | ᱩ 1C69 | ᱽ 1C7D | o 006F | ᱼ 1C7C |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.15.2). When splitting a message text into submessages, there must be no cut right after an SS2. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.25.2	SMS/CBS 7-bit Santali (Ol Chiki script)Alphabet (0x25) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  |  | ᱔ 1C54 |  | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 |  |  | ᱕ 1C55 |  | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 |  |  | ᱖ 1C56 |  | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 |  |  | ᱗ 1C57 |  | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 |  | ^ 005E | ᱘ 1C58 |  | D 0044 | T 0054 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 |  |  | ᱙ 1C59 |  | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 |  |  |  |  | F 0046 | V 0056 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 |  |  |  |  | G 0047 | W 0057 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 |  |  | { 007B |  | H 0048 | X 0058 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 |  | ᱾ 1C7E | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ᱿ 1C7F | ᱑⁄᱔ 1C51,2044,1C54 | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B |  | SS3 ---- | ᱑⁄᱒ 1C51,2044,1C52 | ≠ 2260 | K 004B |  | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C |  | ᱐ 1C50 | ᱓⁄᱔ 1C53,2044,1C54 | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ᱑ 1C51 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E |  | ᱒ 1C52 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ᱓ 1C53 | \ 005C | · 00B7 | O 004F |  | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |