# 5	CBS and USSD Data Coding Scheme

((This subsection should really be moved to the CBS, USSD, MMI standards:  3GPP TS 23.041 Technical realization of the Cell Broadcast Service (CBS), 3GPP TS 23.090 Unstructured Supplementary Service Data (USSD) - Stage 2 or TS 24.090 Unstructured Supplementary Service Data (USSD) - Stage 3, 3GPP TS 22.030 Man-Machine Interface (MMI) of the User Equipment (UE).))

| Bits 7..4 | Use of bits 3..0 (sometimes 5..0) |
| --- | --- |
| ((Neither of these ways of language tagging make much sense; deprecate?)) Otherwise: say “General Data coding indication with language indication”, “Uncompressed” (of course) and “no message class” (and 8-bit is not covered). |  |
| 0000 (0x00..0x0F) 7-bit alphabet as set in the TP User Data Header, language indication for alphabets 0x00, 0x10, or 0x11 | For other alphabets: 0000..1110 are reserved, and group 0000 should not be used. |
|  | 0000 (0x00) German |
|  | 0001 (0x01) English |
|  | 0010 (0x02) Italian |
|  | 0011 (0x03) French |
|  | 0100 (0x04) Spanish |
|  | 0101 (0x05) Dutch |
|  | 0110 (0x06) Swedish |
|  | 0111 (0x07) Danish |
|  | 1000 (0x08) Portuguese |
|  | 1001 (0x09) Finnish |
|  | 1010 (0x0A) Norwegian |
|  | 1011 (0x0B) Greek if alphabet is 0x00 (deprecated); though may be used for alphabet 0x11. |
|  | 1100 (0x0C) Turkish |
|  | 1101 (0x0D) Hungarian |
|  | 1110 (0x0E) Polish |
|  | 1111 (0x0F) Language is unspecified. Common use. Shall be used for 7-bit alphabets other than 0x00, 0x10, 0x11. Continued at group 0010 for alphabet 0x10. |
| 0001 (0x10..0x1F): Language indication in a message prefix | 7-bit alphabet as set in the TP User Data Header 0000 (0x10) Message preceded by language tag. The first 3 characters of the message are a two-character representation of the language encoded according to ISO 639-1 [12], followed by a CR character. The CR character is then followed by (up to 90) 7-bit code units of text. If the alphabet is 0x00 (deprecated) and the language tag is “el<CR>”, then use the Greek special base table for alphabet 0x00, 6.2.1.3, deprecated. (NOTE 1). An MS not supporting the set 7-bit alphabet may present the two character language identifier followed by improperly interpreted user data. NOTE 2: Only two-letter language indications can be used. NOTE 3: The language tag prefix is in the alphabet 0x00, and consists of lowercase letters a-z. |
|  | UCS2/UTF-16BE encoding 0001 (0x11) The character coding is UCS2/UTF-16BE (except for the language tag) and the message is preceded bya two-letter language tag. The full message starts with two 7-bit alphabet (0x00) character representation of the language code encoded according to ISO 639-1 [12]. This is padded to the octet boundary with two bits set to 0 and then followed by (up to 40) 16-bit code units of UTF16BE-encoded message. (NOTE 1). An MS not supporting the UCS2/UTF16BE coding may present the two character language identifier followed by improperly interpreted user data. NOTE 2: Only two-letter language indications can be used. NOTE 3: The language tag prefix is in the 7-bit alphabet 0x00, and consists of lowercase letters a-z, but the message is in UCS2/UTF-16BE. |
|  | 0010..1111 (0x12..0x1F) Reserved |
| 0010 (0x20..0x2F) 7-bit alphabet as set in the TP User Data Header, Language language indication for alphabet 0x10 (continued from group 0000) | For other 7-bit alphabets: 0000..1111 are reserved, and group 0010 should not be used. |
|  | 0000 (0x20) Czech |
|  | 0001..0011 (0x21..0x23) Reserved |
|  | 0100 (0x24) Icelandic |
|  | 0101 (0x25) Romanian |
|  | 0110 (0x26) Albanian |
|  | 0111 (0x27) Catalan |
|  | 1000 (0x28) Croatian |
|  | 1001 (0x29) Slovak |
|  | 1010 (0x2A) Estonian |
|  | 1011 (0x2B) Latvian |
|  | 1100 (0x2C) Lithuanian |
|  | 1101 (0x2D) Vietnamese |
|  | 1110 (0x2E) Irish |
|  | 1111 (0x2F) Tagalog |
| 0011 (0x30..0x3F) | 0000..1111 (0x30..0x3F) Reserved for other languages when using a 7-bit alphabet in future versions of this standard |
| 01xx (0x40 .. 0x7F) | General Data Coding indication |
|  | Bit 5, if set to 0, indicates the text is uncompressed |
|  | Bit 5, if set to 1, indicates the text is compressed using the compression algorithm defined in 3GPP TS 23.042 [13] (strongly deprecated; 3GPP TS 23.042: to withdraw) |
|  | Bit 4, if set to 0, indicates that bits 1 to 0 are reserved and have no message class meaning; interpret as Class 1. |
|  | Bit 4, if set to 1, indicates that bits 1 to 0 have a message class meaning. |
|  | Bit 3 Bit 2 Alphabet: |
|  | 0 0 The 7-bit alphabet as set in the TP User Data Header (default alphabet ID is 0x00) |
|  | 0 1 8 bit data |
|  | 1 0 UCS2/UTF-16BE (16 bit) [10] |
|  | 1 1 Reserved |
|  | Bit 1 Bit 0 Message Class (if bit 4 is set to 1): |
|  | 0 0 Class 0: Immediate. |
|  | 0 1 Class 1: Default meaning: ME-specific. |
|  | 1 0 Class 2: (U)SIM specific message. |
|  | 1 1 Class 3: Default meaning: TE-specific (see 3GPP TS 27.005 [8]) |
| 1000 (0x80..0x8F) | Reserved coding group |
| 1001 (0x90..0x9F) | Message with (secondary?) User Data Header (UDH) structure (prefix?) |
|  | Bit 3 Bit 2 Alphabet: |
|  | 0 0 The 7-bit alphabet as set in the TP User Data Header (default alphabet ID is 0x00) |
|  | 0 1 8 bit data |
|  | 1 0 UCS2/UTF-16BE (16 bit) [10] |
|  | 1 1 Reserved |
|  | Bit 1 Bit 0 Message Class: |
|  | 0 0 Class 0: Immediate. |
|  | 0 1 Class 1: Default meaning: ME-specific. |
|  | 1 0 Class 2: (U)SIM specific message. |
|  | 1 1 Class 3: Default meaning: TE-specific (see 3GPP TS 27.005 [8]) |
| 1010..1100 (0xA0..0xCF) | Reserved coding groups |
| 1101 (0xD0..0xDF) | I1 protocol message defined in 3GPP TS 24.294 [19] USSD when used for I1; reserved coding group otherwise. |
| 1110 (0xE0..0xEF) | Defined by the WAP Forum [15] USSD when used for WAP (obsolete except for MMS(??)); reserved coding group otherwise. |
| 1111 (0xF0..0xFF) | ????????? the 1111 group seems to be an editorial mistake reserved |
|  | Bit 3 Bit 2 Alphabet: |
|  | 0 0 The 7-bit alphabet as set in the TP User Data Header (default alphabet ID is 0x00) |
|  | 0 1 8 bit data |
|  | 1 0 Reserved (UCS2/UTF-16BE (16 bit) [10]…) |
|  | 1 1 Reserved |
|  | Bit 1 Bit 0 Message Class: |
|  | 0 0 Class 0: Immediate. |
|  | 0 1 Class 1: Default meaning: ME-specific. |
|  | 1 0 Class 2: (U)SIM specific message. |
|  | 1 1 Class 3: Default meaning: TE-specific (see 3GPP TS 27.005 [8]) |