6.2.1	SMS/CBS 7-bit GSM default alphabet (0x00) locking shift table

The table below is deprecated and should be used only where it is explicitly required in this document. For Latin script SMS/CBS messages, the new default (Latin) alphabet (reference code 0x10) is preferred over this one. There is a separate (new) alphabet for Greek (0x11).

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | Δ 0394 | SP 0020 | 0 0030 | ¡ 00A1 | P 0050 | ¿ 00BF | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | A 0041 | Q 0051 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | Φ 03A6 | " 0022 | 2 0032 | B 0042 | R 0052 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | Γ 0393 | # 0023 | 3 0033 | C 0043 | S 0053 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | è 00E8 | Λ 039B | ¤ 00A4 | 4 0034 | D 0044 | T 0054 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | é 00E9 | Ω 03A9 | % 0025 | 5 0035 | E 0045 | U 0055 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ù 00F9 | Π 03A0 | & 0026 | 6 0036 | F 0046 | V 0056 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ì 00EC | Ψ 03A8 | ' 0027 | 7 0037 | G 0047 | W 0057 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ò 00F2 | Σ 03A3 | ( 0028 | 8 0038 | H 0048 | X 0058 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ç 00E7 | Θ 0398 | ) 0029 | 9 0039 | I 0049 | Y 0059 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | Ξ 039E | * 002A | : 003A | J 004A | Z 005A | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | Ø 00D8 | SS2 ---- | + 002B | ; 003B | K 004B | Ä 00C4 | k 006B | ä 00E4 |
| 1 | 1 | 0 | 0 | C | ø 00F8 | Æ 00C6 | , 002C | < 003C | L 004C | Ö 00D6 | l 006C | ö 00F6 |
| 1 | 1 | 0 | 1 | D | CR 000D | æ 00E6 | - 002D | = 003D | M 004D | Ñ 00D1 | m 006D | ñ 00F1 |
| 1 | 1 | 1 | 0 | E | Å 00C5 | ß 00DF | . 002E | > 003E | N 004E | Ü 00DC | n 006E | ü 00FC |
| 1 | 1 | 1 | 1 | F | å 00E5 | É 00C9 | / 002F | ? 003F | O 004F | § 00A7 | o 006F | à 00E0 |
| LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is used as a filler (if needed) after the actual SMS/CBS message, as well as in CBS messages after a two-letter language tag in 7-bit messages (if the message heading specifies that there is such a language tag). It should not occur inside the actual message, but if it does, it should be interpreted as if it was an LF. Note that SMS does not have any language tagging mechanism in the protocol, but CBS has two such mechanisms, out of which the second type is required for CBS messages in UTF16BE; neither applies to non-default 7-bit alphabets. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause 6.2.1.1). (Note: locking shift and single shift (SS2) alphabets must be set in synchrony.) |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.1.1	SMS/CBS 7-bit GSM default alphabet (0x00) single shift (SS2) table

The table below is deprecated and should be used only where it is explicitly required in this document. For Latin script SMS/CBS messages, the new default (Latin) alphabet (reference code 0x10) is preferred over this one.

This table (reference code 0x00) is frozen, empty positions will never be filled.

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  |  |  |  | ¦ 007C | 0000 |  | 0000 |
| 0 | 0 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 0 | 2 |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 3 |  |  |  |  |  |  |  |  |
| 0 | 1 | 0 | 0 | 4 |  | ^ 005E |  |  |  |  |  |  |
| 0 | 1 | 0 | 1 | 5 |  |  |  |  |  |  | € 20AC |  |
| 0 | 1 | 1 | 0 | 6 |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 | 1 | 7 |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 8 |  |  | { 007B |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 9 |  |  | } 007D |  |  |  |  |  |
| 1 | 0 | 1 | 0 | A | FF 000C |  |  |  |  |  |  |  |
| 1 | 0 | 1 | 1 | B |  | SS3 ---- |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 | C |  |  |  | [ 005B |  |  |  |  |
| 1 | 1 | 0 | 1 | D | UND |  |  | ~ 007E |  |  |  |  |
| 1 | 1 | 1 | 0 | E |  |  |  | ] 005D |  |  |  |  |
| 1 | 1 | 1 | 1 | F |  |  | \ 005C |  |  |  |  |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). UND: Explicitly undefined. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. On receipt of this code, a receiving entity shall display the 7-bit SS3 and the follow-on code unit as REPLACEMENT CHARACTER (U+FFFD). (There is no SS3 extension table.) |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.1.2	SMS/CBS 7-bit GSM default alphabet (0x00) locking shift table, Greek; DO NOT USE; kept here only for reference with older versions

If the text of the message is deemed to be in “uppercase Greek” (note that only CBS has any form of language tagging), then instead use the following mapping table. The following table is strongly deprecated but listed here for compatibility with earlier versions. Some Latin uppercase letters are replaced by same-looking Greek ones.

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | (el) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | Δ 0394 | SP 0020 | 0 0030 | ¡ 00A1 | Ρ 03A1 | ¿ 00BF | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | Α 0391 | Q 0051 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | Φ 03A6 | " 0022 | 2 0032 | Β 0392 | R 0052 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | Γ 0393 | # 0023 | 3 0033 | C 0043 | S 0053 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | è 00E8 | Λ 039B | ¤ 00A4 | 4 0034 | D 0044 | Τ 03A4 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | é 00E9 | Ω 03A9 | % 0025 | 5 0035 | Ε 0395 | U 0055 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ù 00F9 | Π 03A0 | & 0026 | 6 0036 | F 0046 | V 0056 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ì 00EC | Ψ 03A8 | ' 0027 | 7 0037 | G 0047 | W 0057 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ò 00F2 | Σ 03A3 | ( 0028 | 8 0038 | Η 0397 | Χ 03A7 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ç 00E7 | Θ 0398 | ) 0029 | 9 0039 | Ι 0399 | Υ 03A5 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | Ξ 039E | * 002A | : 003A | J 004A | Ζ 0396 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | Ø 00D8 | SS2 ---- | + 002B | ; 003B | Κ 039A | Ä 00C4 | k 006B | ä 00E4 |
| 1 | 1 | 0 | 0 | C | ø 00F8 | Æ 00C6 | , 002C | < 003C | L 004C | Ö 00D6 | l 006C | ö 00F6 |
| 1 | 1 | 0 | 1 | D | CR 000D | æ 00E6 | - 002D | = 003D | Μ 039C | Ñ 00D1 | m 006D | ñ 00F1 |
| 1 | 1 | 1 | 0 | E | Å 00C5 | ß 00DF | . 002E | > 003E | Ν 039D | Ü 00DC | n 006E | ü 00FC |
| 1 | 1 | 1 | 1 | F | å 00E5 | É 00C9 | / 002F | ? 003F | Ο 039F | § 00A7 | o 006F | à 00E0 |
| LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is used as a filler (if needed) after the actual SMS/CBS message, as well as in CBS messages after a two-letter language tag in 7-bit messages (if the message heading specifies that there is such a language tag). It should not occur inside the actual message, but if it does, it should be interpreted as if it was an LF. Note that SMS does not have any language tagging mechanism in the protocol, but CBS has two such mechanisms, out of which the second type is required for CBS messages in UTF16BE; neither applies to non-default 7-bit alphabets. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause 6.2.1.1). (Note: locking shift and single shift (SS2) alphabets must be set in synchrony.) |  |  |  |  |  |  |  |  |  |  |  |  |