A.4.22	SMS/CBS 7-bit Armenian Alphabet (0x22)

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 |  | SP 0020 | 0 0030 | ա 0561 | ձ 0571 | ց 0581 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | բ 0562 | ղ 0572 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | ՚ 055A | " 0022 | 2 0032 | գ 0563 | ճ 0573 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ՛ 055B | # 0023 | 3 0033 | դ 0564 | մ 0574 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ՟ 055F | ¤ 00A4 | 4 0034 | ե 0565 | յ 0575 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 |  | ֊ 058A | % 0025 | 5 0035 | զ 0566 | ն 0576 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 |  | ՝ 055D | & 0026 | 6 0036 | է 0567 | շ 0577 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 |  | ։ 0589 | ' 0027 | 7 0037 | ը 0568 | ո 0578 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 |  | ՜ 055C | ( 0028 | 8 0038 | թ 0569 | չ 0579 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 |  | ՞ 055E | ) 0029 | 9 0039 | ժ 056A | պ 057A | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A |  | * 002A | : 003A | ի 056B | ջ 057B | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | + 002B | SS2 ---- |  | ; 003B | լ 056C | ռ 057C | k 006B | ւ 0582 |
| 1 | 1 | 0 | 0 | C | − 2212 | ՙ 0559 | , 002C | < 003C | խ 056D | ս 057D | l 006C | փ 0583 |
| 1 | 1 | 0 | 1 | D | CR 000D | ՠ 0560 | - 002D | = 003D | ծ 056E | վ 057E | m 006D | ք 0584 |
| 1 | 1 | 1 | 0 | E |  | ֈ 0588 | . 002E | > 003E | կ 056F | տ 057F | n 006E | օ 0585 |
| 1 | 1 | 1 | 1 | F |  | և 0587 | / 002F | ? 003F | հ 0570 | ր 0580 | o 006F | ֆ 0586 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). The precomposed letter with vowel are included here because the Unicode normalisation NFC will create them from the decomposed form. LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is used as a filler (if needed) after the actual SMS/CBS message, as well as in CBS messages after a two-letter language tag in 7-bit messages (if the message heading specifies that there is such a language tag). It should not occur inside the actual message, but if it does, it should be interpreted as if it was an LF. Note that SMS does not have any language tagging mechanism in the protocol, but CBS has two such mechanisms, out of which the second type is required for CBS messages in UTF16BE; neither applies to non-default 7-bit alphabets. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause B12.1). (Note: locking shift and single shift (SS2) alphabets must be set in synchrony.) |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.22.1	SMS/CBS 7-bit Armenian Alphabet (0x22) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  |  | I 0049 | Q 0051 | ¦ 007C | Հ 0540 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 |  | A 0041 | J 004A | R 0052 | Ա 0531 | Ձ 0541 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 |  | B 0042 | K 004B | S 0053 | Բ 0532 | Ղ 0542 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 |  | C 0043 | L 004C | T 0054 | Գ 0533 | Ճ 0543 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 |  | ^ 005E | M 004D | U 0055 | Դ 0534 | Մ 0544 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 |  | D 0044 | N 004E | V 0056 | Ե 0535 | Յ 0545 | € 20AC | ֏ 058F |
| 0 | 1 | 1 | 0 | 6 |  | E 0045 | O 004F | W 0057 | Զ 0536 | Ն 0546 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 |  | F 0046 | P 0050 | X 0058 | Է 0537 | Շ 0547 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 |  | G 0047 | { 007B | Y 0059 | Ը 0538 | Ո 0548 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 |  | H 0048 | } 007D | Z 005A | Թ 0539 | Չ 0549 | × 00D7 | Ր 0550 |
| 1 | 0 | 1 | 0 | A | FF 000C |  |  | ≈ 2248 | Ժ 053A | Պ 054A | ÷ 00F7 | Ց 0551 |
| 1 | 0 | 1 | 1 | B |  | SS3 ---- |  | ≠ 2260 | Ի 053B | Ջ 054B | ± 00B1 | Ւ 0552 |
| 1 | 1 | 0 | 0 | C |  |  |  | [ 005B | Լ 053C | Ռ 054C | ‰ 2030 | Փ 0553 |
| 1 | 1 | 0 | 1 | D | CSI 009B |  | ⩽ 2A7D | ~ 007E | Խ 053D | Ս 054D | « 00AB | Ք 0554 |
| 1 | 1 | 1 | 0 | E |  |  | ⩾ 2A7E | ] 005D | Ծ 053E | Վ 054E | » 00BB | Օ 0555 |
| 1 | 1 | 1 | 1 | F |  |  | \ 005C | · 00B7 | Կ 053F | Տ 054F | ’ 2019 | Ֆ 0556 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. On receipt of this code, a receiving entity shall display the 7-bit SS3 and the follow-on code unit as REPLACEMENT CHARACTER (U+FFFD). (There is no SS3 extension table.) |  |  |  |  |  |  |  |  |  |  |  |  |