3GPP TSG-CT WG1 Meeting # <MTG_SEQ>	<TDoc#>

# <Location>, <Start_Date> - <End_Date>

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.038 | CR | A.4.25 | rev | - | Current version: | 19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add an SAT (Santali) alphabet (code 0x25) for SMS and CBS when writing in Santali (Ol Chiki script, India). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2026-06-01D |
| Category: | B/C |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The Ol Chiki alphabet is not covered by the current 7-bit alphabets. By using the proposed SAT alphabet, many Santali/Ol Chiki script SMS (and CBS) messages can use fewer ‘submessages’ than when needing to use UCS2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a 7-bit alphabet for (modern) Santali. a-z (and A-Z) are still covered, for parts of the message that may be in English. This completes the set of official scripts in India. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some (not supershort) messages may require more submessages when forced to use UCS-2/UTF-16 than when able to use the proposed encoding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial |  |  |  |  |  |  |  |  |

==============First change==============

See document named A.4.25.docx, which contains the full text of new section A.4.25 (under the new section A.4).

This CR is part of a set of coherent CRs for 3GPP 23.038.

There is a machine readable mapping table (to/from Unicode (ISO/IEC 10646), named 3GPP.23.038-7BIT.SAT.TXT. This will be submitted to Unicode Consortium for publication (cannot promise they will actually publish it).

==============End of change==============