3GPP TSG-CT WG1 Meeting # <MTG_SEQ>	<TDoc#>

# <Location>, <Start_Date> - <End_Date>

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.038 | CR | A.4.10 | rev | - | Current version: | 19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add an EUR (Europe, Latin script) alphabet (code 0x10) to replace the default alphabet for SMS and CBS (USSD should still use 0x00, the default alphabet) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2026-06-01D |
| Category: | B/C |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Many (Latin) characters used for European languages are not covered by the default alphabet, “forcing” the use of UCS-2, lengthening the number of bytes used for a message, and may need more ‘submessages’ for the total message. By using the proposed EUR alphabet, many Latin script SMS (and CBS) messages can use fewer ‘submessages’ than when needing to use UCS2. The default alphabet (code 0x00) is insufficient for many European languages and should only be used for USSD, not for SMS/CBS. The Turkish alphabet (code 0x01) is insufficient even for Turkish, and should not be (is not?) used. Likewise for the Spanish (0x02) and Portuguese (0x03) alphabets. While Greek is sort of supported by the default alphabet (code 0x00), it only covers uppercase and no tone marks (modern Greek is written with tone marks), and it also confuses many Greek letters with Latin letters. A new alphabet for Greek, code 0x11, is proposed separately. Do not use alphabets 0x00, 0x01, 0x02, 0x03 for SMS/CBS. 0x00 can still be used for USSD (mostly used for ‘telephony control codes’). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the default encoding (but excepting Greek, see separate proposal; but use a new code, 0x10) to cover many more Latin script characters used for European languages (and indeed also Vietnamese and Tagalog). Compared to the default encoding, several letters from many European languages (written in the Latin script) are added, providing better coverage for Western European languages as well as Eastern European languages (in the Latin script) plus Vietnamese. Greek letter has been removed; Greek is covered in the proposal for alphabet 0x11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some (not supershort) messages may require more submessages when forced to use UCS-2/UTF-16 than when able to use the proposed encoding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial |  |  |  |  |  |  |  |  |

==============First change==============

See document named A.4.10.docx, which contains the full text of new section A.4.10 (under the new section A.4).

This CR is part of a set of coherent CRs for 3GPP 23.038.

There is a machine readable mappint table (to/from Unicode (ISO/IEC 10646), named 3GPP.23.038-7BIT.EUR.TXT. This will be submitted to Unicode Consortium for publication (cannot promise they will actually publish it).

==============End of change==============