3GPP TSG-CT WG1 Meeting # <MTG_SEQ>	<TDoc#>

# <Location>, <Start_Date> - <End_Date>

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.038 | CR | A.4.11 | rev | - | Current version: | 19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add an EL (Greek) alphabet (code 0x11) to replace the default alphabet for SMS and CBS when writing Greek. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2026-06-01D |
| Category: | B/C |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The Greek alphabet is badly treated in the default alphabet (only uppercase, confused with Latin letters, no monotonic tone mark (let alone polytonic, the orthography for Greek before 1980)), “forcing” the use of UCS-2, lengthening the number of bytes used for a message, and may need more ‘submessages’ for the total message. By using the proposed EL alphabet, many Greek script SMS (and CBS) messages can use fewer ‘submessages’ than when needing to use UCS2. While Greek is sort of supported by the default alphabet (code 0x00), it only covers uppercase and no tone marks (modern Greek is written with tone marks), and it also confuses many Greek letters with Latin letters. A new alphabet for Greek, code 0x11, is proposed here. Do not use alphabets 0x00, 0x01, 0x02, 0x03 for SMS/CBS (at all, especially not for Greek). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completely replace the default alphabet for Greek. Compared to the default encoding, there is no confusion with Latin letters (a-z, A-Z still included), lowercase is covered, monotonic is covered (indeed also polytonic). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some (not supershort) messages may require more submessages when forced to use UCS-2/UTF-16 than when able to use the proposed encoding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial |  |  |  |  |  |  |  |  |

==============First change==============

See document named A.4.11.docx, which contains the full text of new section A.4.11 (under the new section A.4).

This CR is part of a set of coherent CRs for 3GPP 23.038.

There is a machine readable mappint table (to/from Unicode (ISO/IEC 10646), named 3GPP.23.038-7BIT.EL.TXT. This will be submitted to Unicode Consortium for publication (cannot promise they will actually publish it).

==============End of change==============