3GPP TSG-CT WG1 Meeting # <MTG_SEQ>	<TDoc#>

# <Location>, <Start_Date> - <End_Date>

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.038 | CR | C-new | rev | - | Current version: | 19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improve the description of 7-bit alphabet selection for a message. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | 2026-06-01D |
| Category: | D |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The (informative!) description needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | More accurate recommendations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading future 7-bit “alphabet” makers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial |  |  |  |  |  |  |  |  |

==============First change==============

See document named C-new.docx, which contains the new full text of section C.

This CR is part of a set of coherent CRs for 3GPP 23.038.

==============End of change==============