A.4.17	SMS/CBS 7-bit Kannada Alphabet (0x17)

A.4.17.1	SMS/CBS 7-bit Kannada Alphabet (0x17) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ಁ 0C81 | ಐ 0C90 | SP 0020 | 0 0030 | ಬ 0CAC | ಾ 0CBE |  | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ಂ 0C82 | _ 005F | ತ 0CA4 | 1 0031 | ಭ 0CAD | ಿ 0CBF | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ಃ 0C83 | ಒ 0C92 | ಟ 0C9F | 2 0032 | ಮ 0CAE | ೀ 0CC0 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ಅ 0C85 | ಓ 0C93 | # 0023 | 3 0033 | ಯ 0CAF | ು 0CC1 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | ಆ 0C86 | ಔ 0C94 | ಥ 0CA5 | 4 0034 | ರ 0CB0 | ೂ 0CC2 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ಇ 0C87 | ಕ 0C95 | ಠ 0CA0 | 5 0035 | ಱ 0CB1 | ೃ 0CC3 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ಈ 0C88 | ಖ 0C96 | ದ 0CA6 | 6 0036 | ಲ 0CB2 | ೢ 0CE2 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ಉ 0C89 | ಗ 0C97 | ಡ 0CA1 | 7 0037 | ಳ 0CB3 | ೖ 0CD6 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ಊ 0C8A | ಘ 0C98 | ಧ 0CA7 | 8 0038 | ೞ 0CDE | ೆ 0CC6 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ಋ 0C8B | ಙ 0C99 | ಢ 0CA2 | 9 0039 | ವ 0CB5 | ೇ 0CC7 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ಚ 0C9A | * 002A | : 003A | ಸ 0CB8 | ೈ 0CC8 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ಌ 0C8C | SS2 ---- | + 002B | ; 003B | ಷ 0CB7 | ೕ 0CD5 | k 006B |  |
| 1 | 1 | 0 | 0 | C |  | ಛ 0C9B | , 002C | ಣ 0CA3 | ಶ 0CB6 | ೊ 0CCA | l 006C |  |
| 1 | 1 | 0 | 1 | D | CR 000D | ಜ 0C9C | ನ 0CA8 | ಪ 0CAA | ಹ 0CB9 | ೋ 0CCB | m 006D |  |
| 1 | 1 | 1 | 0 | E | ಎ 0C8E | ಝ 0C9D | . 002E | ಫ 0CAB | ಼ 0CBC | ೌ 0CCC | n 006E | ॰ 0970 |
| 1 | 1 | 1 | 1 | F | ಏ 0C8F | ಞ 0C9E | / 002F | ? 003F | ್‌ 0CCD,200C | ್ 0CCD | o 006F |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.17.2). When splitting a message text into submessages, there must be no cut right after an SS2. 0CCD KANNADA SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+0CCD is a combining character with nominal glyph. In Kannada, conjuncts are often formed by using a subscript form of the second consonant. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.17.2	SMS/CBS 7-bit Kannada Alphabet (0x17) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ೪ 0CEA | ೠ 0CE0 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ೫ 0CEB | ೡ 0CE1 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ೬ 0CEC | ೄ 0CC4 | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ೭ 0CED | ೣ 0CE3 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ೮ 0CEE |  | D 0044 | T 0054 | ” 201D | ಽ 0CBD |
| 0 | 1 | 0 | 1 | 5 | " 0022 |  | ೯ 0CEF |  | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | € 20AC | ಄ 0C84 |  |  | F 0046 | V 0056 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B | ‍್ 200D, 0CCD | H 0048 | X 0058 | ° 00B0 | ೝ 0CDD |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ೧⁄೪ 0CE7,2044,0CEA | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ೧⁄೨ 0CE7,2044,0CE8 | ≠ 2260 | K 004B | ೱ 0CF1 | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ೦ 0CE6 | ೩⁄೪ 0CE9,2044,0CEA | [ 005B | L 004C | ೲ 0CF2 | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ೧ 0CE7 | ⩽ 2A7D | ~ 007E | M 004D | ೱ‌ 0CF1, 200C | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ೨ 0CE8 | ⩾ 2A7E | ] 005D | N 004E | ೲ‌ 0CF2, 200C | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ೩ 0CE9 | \ 005C | · 00B7 | O 004F | ೳ 0CF3 | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |