A.4.19	SMS/CBS 7-bit Oriya/Odia Alphabet (0x19)

A.4.19.1	SMS/CBS 7-bit Oriya/Odia Alphabet (0x19) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ଁ 0B01 | ଐ 0B10 | SP 0020 | 0 0030 | ବ 0B2C | ା 0B3E | ୕ 0B55 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ଂ 0B02 | _ 005F | ତ 0B24 | 1 0031 | ଭ 0B2D | ି 0B3F | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ଃ 0B03 |  | ଟ 0B1F | 2 0032 | ମ 0B2E | ୀ 0B40 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ଅ 0B05 | ଓ 0B13 | # 0023 | 3 0033 | ଯ 0B2F | ୁ 0B41 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | ଆ 0B06 | ଔ 0B14 | ଥ 0B25 | 4 0034 | ର 0B30 | ୂ 0B42 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ଇ 0B07 | କ 0B15 | ଠ 0B20 | 5 0035 |  | ୃ 0B43 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ଈ 0B08 | ଖ 0B16 | ଦ 0B26 | 6 0036 | ଲ 0B32 | ୢ 0B62 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ଉ 0B09 | ଗ 0B17 | ଡ 0B21 | 7 0037 | ଳ 0B33 | ୖ 0B56 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ଊ 0B0A | ଘ 0B18 | ଧ 0B27 | 8 0038 |  | େ 0B47 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ଋ 0B0B | ଙ 0B19 | ଢ 0B22 | 9 0039 | ଵ 0B35 |  | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ଚ 0B1A | * 002A | : 003A | ସ 0B38 | ୈ 0B48 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ଌ 0B0C | SS2 ---- | + 002B | ; 003B | ଷ 0B37 | ୗ 0B57 | k 006B |  |
| 1 | 1 | 0 | 0 | C |  | ଛ 0B1B | , 002C | ଣ 0B23 | ଶ 0B36 |  | l 006C |  |
| 1 | 1 | 0 | 1 | D | CR 000D | ଜ 0B1C | ନ 0B28 | ପ 0B2A | ହ 0B39 | ୋ 0B4B | m 006D |  |
| 1 | 1 | 1 | 0 | E | ଏ 0B0F | ଝ 0B1D | . 002E | ଫ 0B2B | ଼ 0B3C | ୌ 0B4C | n 006E | ॰ 0970 |
| 1 | 1 | 1 | 1 | F |  | ଞ 0B1E | / 002F | ? 003F | ୍‌ 0B4D,200C | ୍ 0B4D | o 006F |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.19.2). When splitting a message text into submessages, there must be no cut right after an SS2. U+0B4D: ORIYA SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+0B4D is a combining character with nominal glyph. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.19.2	SMS/CBS 7-bit Oriya/Odia Alphabet (0x19) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ୪ 0B6A | ୠ 0B60 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ୫ 0B6B | ୡ 0B61 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ୬ 0B6C | ୄ 0B44 | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ୭ 0B6D | ୣ 0B63 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ୮ 0B6E |  | D 0044 | T 0054 | ” 201D | ଽ 0B3D |
| 0 | 1 | 0 | 1 | 5 | " 0022 |  | ୯ 0B6F | ୓ 0B53 | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | € 20AC |  | ୟ 0B5F | ୔ 0B54 | F 0046 | V 0056 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 | ୱ 0B71 |  | G 0047 | W 0057 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B | ‍୍ 200D,0B4D | H 0048 | X 0058 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ୧⁄୪ 0B67,2044,0B6A | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ୧⁄୨ 0B67,2044,0B68 | ≠ 2260 | K 004B |  | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ୦ 0B66 | ୩⁄୪ 0B69,2044,0B6A | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ୧ 0B67 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ୨ 0B68 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ୩ 0B69 | \ 005C | · 00B7 | O 004F |  | ’ 2019 | ୰ 0B70 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LINE FEED (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |