A.4.11	SMS/CBS 7-bit Greek alphabet (0x11)

This alphabet for Greek should be preferred over the Greek reading of the 7-bit default alphabet (0x00). This alphabet supports Monotonic Greek with both upper- and lowercase. Polytonic Greek is supported using combining characters.

A.4.11.2	SMS/CBS 7-bit Greek alphabet (0x11) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | − 2212 | SP 0020 | 0 0030 | ͅ 0345 | μ 03BC | ς 03C2 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | α 03B1 | ν 03BD | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | ̀ 0300 | " 0022 | 2 0032 | ά 03AC | ξ 03BE | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ́ 0301 | # 0023 | 3 0033 | β 03B2 | ο 03BF | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ͂ 0342 | € 20AC | 4 0034 | γ 03B3 | ό 03CC | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ϝ 03DD | ̓ 0313 | % 0025 | 5 0035 | δ 03B4 | π 03C0 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ϵ 03F5 | ̔ 0314 | & 0026 | 6 0036 | ε 03B5 | ρ 03C1 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ϵ́ 03F5,0301 | ̈ 0308 | ' 0027 | 7 0037 | έ 03AD | σ 03C3 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ϲ 03F2 | ̈́ 0344 | ( 0028 | 8 0038 | ζ 03B6 | τ 03C4 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ϛ 03DB | ̆ 0306 | ) 0029 | 9 0039 | η 03B7 | υ 03C5 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ̄ 0304 | * 002A | : 003A | ή 03AE | ύ 03CD | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ϙ 03D9 | SS2 ---- | + 002B | ; 003B | θ 03B8 | φ 03C6 | k 006B | ʹ 0374 |
| 1 | 1 | 0 | 0 | C | ϟ 03DF | ϊ 03CA | , 002C | < 003C | ι 03B9 | χ 03C7 | l 006C | ͵ 0375 |
| 1 | 1 | 0 | 1 | D | CR 000D | ΐ 0390 | - 002D | = 003D | ί 03AF | ψ 03C8 | m 006D |  |
| 1 | 1 | 1 | 0 | E | ϗ 03D7 | ϋ 03CB | . 002E | > 003E | κ 03BA | ω 03C9 | n 006E |  |
| 1 | 1 | 1 | 1 | F | ϡ 03E1 | ΰ 03B0 | / 002F | ? 003F | λ 03BB | ώ 03CE | o 006F |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.11.2). (Note: setting, in the SMS and CBS protocols, base or extension table with reference value 0x10 or larger automatically sets the other tables with the same reference value.) When splitting a message text into submessages, there must be no cut right after an SS2. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.11.2	SMS/CBS 7-bit Greek alphabet (0x11) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  | ¦ 007C | I 0049 | Q 0051 | ̌ 030C | Μ 039C | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 |  | A 0041 | J 004A | R 0052 | Α 0391 | Ν 039D | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 |  | B 0042 | K 004B | S 0053 | Ά 0386 | Ξ 039E | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 |  | C 0043 | L 004C | T 0054 | Β 0392 | Ο 039F | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 |  | ^ 005E | M 004D | U 0055 | Γ 0393 | Ό 038C | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | Ϝ 03DC | D 0044 | N 004E | V 0056 | Δ 0394 | Π 03A0 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 |  | E 0045 | O 004F | W 0057 | Ε 0395 | Ρ 03A1 |  |  |
| 0 | 1 | 1 | 1 | 7 |  | F 0046 | P 0050 | X 0058 | Έ 0388 | Σ 03A3 |  |  |
| 1 | 0 | 0 | 0 | 8 | Ϲ 03F9 | G 0047 | { 007B | Y 0059 | Ζ 0396 | Τ 03A4 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | Ϛ 03DA | H 0048 | } 007D | Z 005A | Η 0397 | Υ 03A5 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ̓ 0343 |  | ≈ 2248 | Ή 0389 | Ύ 038E | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | Ϙ 03D8 | SS3 ---- |  | ≠ 2260 | Θ 0398 | Φ 03A6 | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | Ϟ 03DE | Ϊ 03AA |  | [ 005B | Ι 0399 | Χ 03A7 | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | Ϊ́ 0399,0344 | ⩽ 2A7D | ~ 007E | Ί 038A | Ψ 03A8 | « 00AB | ; 037E |
| 1 | 1 | 1 | 0 | E | Ϗ 03CF | Ϋ 03AB | ⩾ 2A7E | ] 005D | Κ 039A | Ω 03A9 | » 00BB | · 0387 |
| 1 | 1 | 1 | 1 | F | Ϡ 03E0 | Ϋ́ 03A5,0344 | \ 005C | · 00B7 | Λ 039B | Ώ 038F | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). A final recipient of the SMS/CBS message which does not have pagination shall convert FF to LF. CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another (SS3) extension table. When splitting a message text into submessages, there must be no cut right after an SS3. |  |  |  |  |  |  |  |  |  |  |  |  |