A.4.15	SMS/CBS 7-bit Gujarati Alphabet (0x15)

A.4.15.1	SMS/CBS 7-bit Gujarati Alphabet (0x15) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ઁ 0A81 | ઐ 0A90 | SP 0020 | 0 0030 | બ 0AAC | ા 0ABE | ૐ 0AD0 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ં 0A82 | _ 005F | ત 0AA4 | 1 0031 | ભ 0AAD | િ 0ABF | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ઃ 0A83 |  | ટ 0A9F | 2 0032 | મ 0AAE | ી 0AC0 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | અ 0A85 | ઓ 0A93 | # 0023 | 3 0033 | ય 0AAF | ુ 0AC1 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | આ 0A86 | ઔ 0A94 | થ 0AA5 | 4 0034 | ર 0AB0 | ૂ 0AC2 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ઇ 0A87 | ક 0A95 | ઠ 0AA0 | 5 0035 |  | ૃ 0AC3 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ઈ 0A88 | ખ 0A96 | દ 0AA6 | 6 0036 | લ 0AB2 | ૢ 0AE2 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ઉ 0A89 | ગ 0A97 | ડ 0AA1 | 7 0037 | ળ 0AB3 | ૅ 0AC5 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ઊ 0A8A | ઘ 0A98 | ધ 0AA7 | 8 0038 |  | ે 0AC7 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ઋ 0A8B | ઙ 0A99 | ઢ 0AA2 | 9 0039 | વ 0AB5 |  | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ચ 0A9A | * 002A | : 003A | સ 0AB8 | ૈ 0AC8 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ઌ 0A8C | SS2 ---- | + 002B | ; 003B | ષ 0AB7 | ૉ 0AC9 | k 006B |  |
| 1 | 1 | 0 | 0 | C | ઍ 0A8D | છ 0A9B | , 002C | ણ 0AA3 | શ 0AB6 |  | l 006C |  |
| 1 | 1 | 0 | 1 | D | CR 000D | જ 0A9C | ન 0AA8 | પ 0AAA | હ 0AB9 | ો 0ACB | m 006D |  |
| 1 | 1 | 1 | 0 | E | એ 0A8F | ઝ 0A9D | . 002E | ફ 0AAB | ઼ 0ABC | ૌ 0ACC | n 006E | ૰ 0AF0 |
| 1 | 1 | 1 | 1 | F |  | ઞ 0A9E | / 002F | ? 003F |  | ્ 0ACD | o 006F |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.15.2). (Note: setting, in the SMS and CBS protocols, base or extension table with reference value 0x10 or larger automatically sets the other tables with the same reference value.) When splitting a message text into submessages, there must be no cut right after an SS2. 0ACD GUJARATI SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+0ACD is a combining character with nominal glyph. In Gujarati, conjuncts are often formed by using the half form of the first consonant, which also may be used final consonants. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.15.2	SMS/CBS 7-bit Gujarati Alphabet (0x15) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ૪ 0AEA | ૠ 0AE0 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ૫ 0AEB | ૡ 0AE1 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ૬ 0AEC | ૄ 0AC4 | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ૭ 0AED | ૣ 0AE3 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ૮ 0AEE |  | D 0044 | T 0054 | ” 201D | ઽ 0ABD |
| 0 | 1 | 0 | 1 | 5 | " 0022 | ૱ 0AF1 | ૯ 0AEF |  | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | € 20AC |  |  |  | F 0046 | V 0056 | Ω 03A9 | ઑ 0A91 |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC | ૹ 0AF9 |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B |  | H 0048 | X 0058 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D |  | I 0049 | Y 0059 | × 00D7 | ૽ 0AFD |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ૧⁄૪ 0AE7,2044,0AEA | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 | ૾ 0AFE |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ૧⁄૨ 0AE7,2044,0AE8 | ≠ 2260 | K 004B |  | ± 00B1 | ૿ 0AFF |
| 1 | 1 | 0 | 0 | C | ! 0021 | ૦ 0AE6 | ૩⁄૪ 0AE9,2044,0AEA | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ૧ 0AE7 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB | ૻ 0AFB |
| 1 | 1 | 1 | 0 | E | - 002D | ૨ 0AE8 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB | ૼ 0AFC |
| 1 | 1 | 1 | 1 | F |  | ૩ 0AE9 | \ 005C | · 00B7 | O 004F |  | ’ 2019 | ૺ 0AFA |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |