A.4.21	SMS/CBS 7-bit Sinhala Alphabet (0x21)

A.4.21.1	SMS/CBS 7-bit Sinhala Alphabet (0x21) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ඁ 0D81 | ඓ 0D93 | SP 0020 | 0 0030 | බ 0DB6 | ා 0DCF |  | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ං 0D82 |  | ත 0DAD | 1 0031 | භ 0DB7 | ි 0DD2 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ඃ 0D83 | ඔ 0D94 | ට 0DA7 | 2 0032 | ම 0DB8 | ී 0DD3 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | අ 0D85 | ඕ 0D95 | # 0023 | 3 0033 | ය 0DBA | ු 0DD4 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | ආ 0D86 | ඖ 0D96 | ථ 0DAE | 4 0034 | ර 0DBB | ූ 0DD6 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ඉ 0D89 | ක 0D9A | ඨ 0DA8 | 5 0035 |  | ෘ 0DD8 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ඊ 0D8A | ඛ 0D9B | ද 0DAF | 6 0036 | ල 0DBD | ෟ 0DDF | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | උ 0D8B | ග 0D9C | ඩ 0DA9 | 7 0037 | ළ 0DC5 |  | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ඌ 0D8C | ඝ 0D9D | ධ 0DB0 | 8 0038 |  | ෙ 0DD9 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ඍ 0D8D | ඞ 0D9E | ඪ 0DAA | 9 0039 | ව 0DC0 | ේ 0DDA | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ච 0DA0 | * 002A | : 003A | ස 0DC3 | ෛ 0DDB | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ඏ 0D8F | SS2 ---- | + 002B | ; 003B | ෂ 0DC2 |  | k 006B | ඟ 0D9F |
| 1 | 1 | 0 | 0 | C |  | ඡ 0DA1 | , 002C | ණ 0DAB | ශ 0DC1 | ො 0DDC | l 006C | ඥ 0DA5 |
| 1 | 1 | 0 | 1 | D | CR 000D | ජ 0DA2 | න 0DB1 | ප 0DB4 | හ 0DC4 | ෝ 0DDD | m 006D | ඦ 0DA6 |
| 1 | 1 | 1 | 0 | E | එ 0D91 | ඣ 0DA3 | . 002E | ඵ 0DB5 |  | ෞ 0DDE | n 006E | ඬ 0DAC |
| 1 | 1 | 1 | 1 | F | ඒ 0D92 | ඤ 0DA4 | / 002F | ? 003F |  | ් 0DCA | o 006F | ඳ 0DB3 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.21.2). (Note: setting, in the SMS and CBS protocols, base or extension table with reference value 0x10 or larger automatically sets the other tables with the same reference value.) When splitting a message text into submessages, there must be no cut right after an SS2. U+0DCA: SINHALA SIGN AL-LAKUNA: Virama; This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+0DCA is a combining character with nominal glyph. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.21.2	SMS/CBS 7-bit Sinhala Alphabet (0x21) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ෪ 0DEA | ෲ 0DF2 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ෫ 0DEB | ෳ 0DF3 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ෬ 0DEC | ඎ 0D8E | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ෭ 0DED | ඐ 0D90 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ෮ 0DEE | ැ 0DD0 | D 0044 | T 0054 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | " 0022 |  | ෯ 0DEF | ෑ 0DD1 | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | € 20AC |  |  | ඇ 0D87 | F 0046 | V 0056 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 | ෴ 0DF4 | ඈ 0D88 | G 0047 | W 0057 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B | ඹ 0DB9 | H 0048 | X 0058 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D | ෆ 0DC6 | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ෧⁄෪ 0DE7,2044,0DEA | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ෧⁄෨ 0DE7,2044,0DE8 | ≠ 2260 | K 004B |  | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ෦ 0DE6 | ෩⁄෪ 0DE9,2044,0DEA | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ෧ 0DE7 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ෨ 0DE8 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ෩ 0DE9 | \ 005C | · 00B7 | O 004F |  | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044: FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |