A.4.23	SMS/CBS 7-bit Georgian Alphabet (0x23)

A.4.23.1	SMS/CBS 7-bit Georgian Alphabet (0x23) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | − 2212 | SP 0020 | 0 0030 | ა 10D0 | რ 10E0 | ჰ 10F0 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | ბ 10D1 | ს 10E1 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | ჶ 10F6 | " 0022 | 2 0032 | გ 10D2 | ტ 10E2 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ჷ 10F7 | # 0023 | 3 0033 | დ 10D3 | უ 10E3 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ჸ 10F8 | € 20AC | 4 0034 | ე 10D4 | ფ 10E4 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ჻ 10FB | ჹ 10F9 | % 0025 | 5 0035 | ვ 10D5 | ქ 10E5 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ֏ 058F | ჺ 10FA | & 0026 | 6 0036 | ზ 10D6 | ღ 10E6 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ₾ 20BE | ჽ 10FD | ' 0027 | 7 0037 | თ 10D7 | ყ 10E7 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ₴ 20B4 | ჾ 10FE | ( 0028 | 8 0038 | ი 10D8 | შ 10E8 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 |  | ჿ 10FF | ) 0029 | 9 0039 | კ 10D9 | ჩ 10E9 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ჼ 10FC | * 002A | : 003A | ლ 10DA | ც 10EA | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B |  | SS2 ---- | + 002B | ; 003B | მ 10DB | ძ 10EB | k 006B | ჱ 10F1 |
| 1 | 1 | 0 | 0 | C |  |  | , 002C | < 003C | ნ 10DC | წ 10EC | l 006C | ჲ 10F2 |
| 1 | 1 | 0 | 1 | D | CR 000D |  | - 002D | = 003D | ო 10DD | ჭ 10ED | m 006D | ჳ 10F3 |
| 1 | 1 | 1 | 0 | E |  |  | . 002E | > 003E | პ 10DE | ხ 10EE | n 006E | ჴ 10F4 |
| 1 | 1 | 1 | 1 | F |  |  | / 002F | ? 003F | ჟ 10DF | ჯ 10EF | o 006F | ჵ 10F5 |
| LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.23.2). (Note: setting, in the SMS and CBS protocols, base or extension table with reference value 0x10 or larger automatically sets the other tables with the same reference value.) When splitting a message text into submessages, there must be no cut right after an SS2. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.23.2	SMS/CBS 7-bit Georgian Alphabet (0x23) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  | ¦ 007C | I 0049 | Q 0051 | Ა 1C90 | Რ 1CA0 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 |  | A 0041 | J 004A | R 0052 | Ბ 1C91 | Ს 1CA1 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | Ჶ 1CB6 | B 0042 | K 004B | S 0053 | Გ 1C92 | Ტ 1CA2 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | Ჷ 1CB7 | C 0043 | L 004C | T 0054 | Დ 1C93 | Უ 1CA3 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | Ჸ 1CB8 | ^ 005E | M 004D | U 0055 | Ე 1C94 | Ფ 1CA4 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | Ჹ 1CB9 | D 0044 | N 004E | V 0056 | Ვ 1C95 | Ქ 1CA5 | ¤ 00A4 | ՚ 055A |
| 0 | 1 | 1 | 0 | 6 | Ჺ 1CBA | E 0045 | O 004F | W 0057 | Ზ 1C96 | Ღ 1CA6 | Ω 03A9 | ՛ 055B |
| 0 | 1 | 1 | 1 | 7 | Ჽ 1CBD | F 0046 | P 0050 | X 0058 | Თ 1C97 | Ყ 1CA7 | μ 03BC | ՟ 055F |
| 1 | 0 | 0 | 0 | 8 | Ჾ 1CBE | G 0047 | { 007B | Y 0059 | Ი 1C98 | Შ 1CA8 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | Ჿ 1CBF | H 0048 | } 007D | Z 005A | Კ 1C99 | Ჩ 1CA9 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ֊ 058A |  | ≈ 2248 | Ლ 1C9A | Ც 1CAA | ÷ 00F7 | Ჰ 1CB0 |
| 1 | 0 | 1 | 1 | B |  | SS3 ---- |  | ≠ 2260 | Მ 1C9B | Ძ 1CAB | ± 00B1 | Ჱ 1CB1 |
| 1 | 1 | 0 | 0 | C |  | ՝ 055D |  | [ 005B | Ნ 1C9C | Წ 1CAC | ‰ 2030 | Ჲ 1CB2 |
| 1 | 1 | 0 | 1 | D | CSI 009B | ։ 0589 | ⩽ 2A7D | ~ 007E | Ო 1C9D | Ჭ 1CAD | « 00AB | Ჳ 1CB3 |
| 1 | 1 | 1 | 0 | E |  | ՜ 055C | ⩾ 2A7E | ] 005D | Პ 1C9E | Ხ 1CAE | » 00BB | Ჴ 1CB4 |
| 1 | 1 | 1 | 1 | F |  | ՞ 055E | \ 005C | · 00B7 | Ჟ 1C9F | Ჯ 1CAF | ’ 2019 | Ჵ 1CB5 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. |  |  |  |  |  |  |  |  |  |  |  |  |