A.4.24	SMS/CBS 7-bit Ukrainian (Cyrillic) Alphabet (0x24)

A.4.24.1	SMS/CBS 7-bit Ukrainian (Cyrillic) Alphabet (0x24) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | − 2212 | SP 0020 | 0 0030 | г 0433 | м 043C |  | p 0070 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | _ 005F | ! 0021 | 1 0031 | ґ 0491 | н 043D | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | ₴ 20B4 | " 0022 | 2 0032 | д 0434 | њ 045A | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 |  | # 0023 | 3 0033 | ђ 0452 | о 043E | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 |  | € 20AC | 4 0034 | е 0435 | п 043F | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ѕ 0455 |  | % 0025 | 5 0035 | є 0454 | р 0440 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ѳ 0473 |  | & 0026 | 6 0036 | ж 0436 | с 0441 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ѯ 046F |  | ' 0027 | 7 0037 | з 0437 | т 0442 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ҁ 0481 |  | ( 0028 | 8 0038 | и 0438 | ћ 045B | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ѱ 0471 |  | ) 0029 | 9 0039 | і 0456 | у 0443 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ъ 044A | * 002A | : 003A | ї 0457 | ф 0444 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ѡ 0461 | SS2 ---- | + 002B | ; 003B | й 0439 | х 0445 | k 006B | щ 0449 |
| 1 | 1 | 0 | 0 | C | ѧ 0467 | ѣ 0463 | , 002C | < 003C | ј 0458 | ц 0446 | l 006C | ѫ 046B |
| 1 | 1 | 0 | 1 | D | CR 000D | а 0430 | - 002D | = 003D | к 043A | ч 0447 | m 006D | ь 044C |
| 1 | 1 | 1 | 0 | E | ֏ 058F | б 0431 | . 002E | > 003E | л 043B | џ 045F | n 006E | ю 044E |
| 1 | 1 | 1 | 1 | F | ₾ 20BE | в 0432 | / 002F | ? 003F | љ 0459 | ш 0448 | o 006F | я 044F |
| LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.24.2). (Note: setting, in the SMS and CBS protocols, base or extension table with reference value 0x10 or larger automatically sets the other tables with the same reference value.) When splitting a message text into submessages, there must be no cut right after an SS2. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.24.2	SMS/CBS 7-bit Ukrainian (Cyrillic) Alphabet (0x24) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  | ¦ 007C | I 0049 | Q 0051 | Г 0413 | М 041C | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 |  | A 0041 | J 004A | R 0052 | Ґ 0490 | Н 041D | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 |  | B 0042 | K 004B | S 0053 | Д 0414 | Њ 040A | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 |  | C 0043 | L 004C | T 0054 | Ђ 0402 | О 041E | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 |  | ^ 005E | M 004D | U 0055 | Е 0415 | П 041F | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | Ѕ 0405 | D 0044 | N 004E | V 0056 | Є 0404 | Р 0420 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | Ѳ 0472 | E 0045 | O 004F | W 0057 | Ж 0416 | С 0421 |  |  |
| 0 | 1 | 1 | 1 | 7 | Ѯ 046E | F 0046 | P 0050 | X 0058 | З 0417 | Т 0422 |  |  |
| 1 | 0 | 0 | 0 | 8 | Ҁ 0480 | G 0047 | { 007B | Y 0059 | И 0418 | Ћ 040B | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | Ѱ 0470 | H 0048 | } 007D | Z 005A | І 0406 | У 0423 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | Ъ 042A |  | ≈ 2248 | Ї 0407 | Ф 0424 | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | Ѡ 0460 | SS3 ---- |  | ≠ 2260 | Й 0419 | Х 0425 | ± 00B1 | Щ 0429 |
| 1 | 1 | 0 | 0 | C | Ѧ 0466 | Ѣ 0462 |  | [ 005B | Ј 0408 | Ц 0426 | ‰ 2030 | Ѫ 046A |
| 1 | 1 | 0 | 1 | D | CSI 009B | А 0410 | ⩽ 2A7D | ~ 007E | К 041A | Ч 0427 | « 00AB | Ь 042C |
| 1 | 1 | 1 | 0 | E | ҃ 0483 | Б 0411 | ⩾ 2A7E | ] 005D | Л 041B | Џ 040F | » 00BB | Ю 042E |
| 1 | 1 | 1 | 1 | F | ҂ 0482 | В 0412 | \ 005C | · 00B7 | Љ 0409 | Ш 0428 | ’ 2019 | Я 042F |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. |  |  |  |  |  |  |  |  |  |  |  |  |